Linear Actuator Control System Report

1. Introduction

The purpose of this task was to design and implement a control system for a linear actuator powered by a 48 V DC supply. The actuator must move upward or downward depending on the polarity of the supply voltage. Two 24 V digital controller outputs are used for directional control. Additional requirements included implementing safety interlock logic, using RS-485 feedback to monitor the actuator position, and achieving automatic reversal when limits are reached.

2. Circuit Design

A relay-based polarity reversal circuit (H-bridge style) was designed using two SPDT relays. When the UP command is active, Relay K1 energizes and applies +48 V to the actuator in one polarity. When the DOWN command is active, Relay K2 energizes and reverses the polarity. A transistor interface is used so the 24 V controller signals can drive the relay coils.

Safety components include a fuse for overcurrent protection and flyback diodes across each relay coil to protect against voltage spikes.

- OUT_0.0 → K1 → Moves actuator UP

- OUT_0.1 → K2 → Moves actuator DOWN

A software interlock ensures that both relays are never active at the same time.

A complete schematic was created in KiCad and exported as part of this submission.

3. Programming Logic

The actuator movement logic is based on live position feedback from an RS-485 sensor. The system starts by moving upward, and once the actuator reaches the upper limit, it stops and changes direction. The same process occurs at the lower limit. The system keeps running in a loop and counts how many complete cycles are executed.

A safety rule stops movement if both relay outputs become active simultaneously.

4. Position Control

A second position-based control mode was implemented. In this mode, the actuator moves until it reaches a user-defined target position. If the current position is below the target, the actuator moves upward. If the current position is above the target, it moves downward. Motion stops automatically when the position is within an acceptable tolerance range.

Limit protection ensures the actuator cannot move past its mechanical boundaries.

5. Safety Measures

- Software interlock prevents both relays from energizing at the same time.

- Back-EMF protection diodes safeguard relay driver circuitry.

- Fuse protects wiring and actuator hardware.

- Fault detection stops motion if invalid feedback is detected.

6. Submitted Files

| File Name | Description |
| --- | --- |
| Linear_Actuator_Schematic.pdf | Circuit design |
| Actuator_Programming_Logic.txt | Control logic pseudocode |
| Actuator_Position_Control.txt | Position-based control logic |

7. Conclusion

The complete actuator control solution meets all required specifications including polarity reversal, RS-485 feedback, automatic motion control, safety interlocking, and structured program logic. The system is stable, expandable, and suitable for integration into larger automation platforms.